Anna Comments Review 1-22-2025

- It is not purely joint space or state space. These loading scenarios consist of tendon actuation variables [tau1, tau2, tau3], and force variables [F1, s1, theta1, F2, s2, theta2].

- The forces are radially applied and shown in the diagram.

- Later in the method I discuss normalization.

- Random electrical fluctuations.

- The hysteresis I experimentally tested was due to the material properties of the robot (plasticity, internal friction, 3D printing artifacts). What this means is that let’s say you change your load from 0.5 N to 1 N and then back down to 0.5 N again. The sensor will not return to the same original value because the unloading and de-loading plots are different.

  - Based on subgroup meeting, I should probably also see what uncertainty I get from the order in which apply my tendon tensions and forces. This uncertainty is caused by tendon friction

  - If I had no tendon friction and no viscoelasticity, then order would theoretically not matter.

  - Without standardizing my loading sequence order, I would get random unpredictable uncertainty in my wrench measurement due to tendon friction. By standardizing the sequence, I convert the random uncertainty into a more predictable systematic bias.

  - Problem, If I apply tendons first and then force, force ill change tendons. Vice versa for other way around.

- It is not, maybe I should use another symbol.

- First we scale the input variables [F1, s1, theta1, F2, s2, theta2] by their max range so that a full range changes in input variable have similar scales. This prevents a single input from dominating the Jacobian. We then scale the output variables [Tx, Ty, Tz, Fx, Fy, Fz], dividing each one by its expected random noise, converting the output to a signal to noise ratio. This puts all the output on a comparable scale allowing us to compare which signals are trustworthy and which are just buried in noise.

- SNR is another function.