Force estimation:

Should we increase upper bound on force to 3N?

- Increasing this upper bound will likely reduce force estimation accuracy as the optimizer will have a larger space to search over.

- Below are ranges used in similar force-sensing papers:

  - 0.27 N to 1.97 N [1]

  - 0.5 N to 1.0 N [2]

  - 0.5 N to 3.5 N [3]

- ATI overload torque: 19 Nm

- ATI accurate sensing range: 0 to 1 Nm

- Worst-case torque: (1.5 N + 1.5 N) * (0.09 N + 0.2 N) = 0.87 Nm

- Conclusion: 0.3 N to 1.5 N is suitable range because it is comparable to other works and remains in the ATI accurate sensing range.

Should we divide design variables [F1, s1,…]  by ref value to get roughly unit values for each variable?

- I will use ‘TypicalX’ in MATLAB lsqnonlin to account for this scaling.

Design Optimization:

What does the force to wrench Jacobian look like?

Jraw=∂W∂F=∂Tx∂F1∂Tx∂s1∂Tx∂θ1∂Tx∂F2∂Tx∂s2∂Tx∂θ2∂Ty∂F1∂Ty∂s1∂Ty∂θ1∂Ty∂F2∂Ty∂s2∂Ty∂θ2∂Tz∂F1∂Tz∂s1∂Tz∂θ1∂Tz∂F2∂Tz∂s2∂Tz∂θ2∂Fx∂F1∂Fx∂s1∂Fx∂θ1∂Fx∂F2∂Fx∂s2∂Fx∂θ2∂Fy∂F1∂Fy∂s1∂Fy∂θ1∂Fy∂F2∂Fy∂s2∂Fy∂θ2∂Fz∂F1∂Fz∂s1∂Fz∂θ1∂Fz∂F2∂Fz∂s2∂Fz∂θ2∈R6x6

Should we switch to optimization under uncertainty?

- Monte Carlo (noise-injected inverse simulation)
Add Gaussian noise to the proximal wrench and repeatedly run the full inverse force-sensing model. This reveals how the estimator performs under realistic measurement uncertainty. This method directly mirrors real-world behavior and is an excellent optimization method. I previously implemented this approach in MAE 270 for a single planar force and obtained good results. However, my current force-sensing model is significantly more complex, making thousands of full inverse simulations computationally impractical. This motivated the search for faster alternatives.

- SVD-based sensitivity metric
The SVD approach evaluates sensitivity using only the forward model, making it computationally efficient. However, it provides only an approximation of error propagation. The result is a dimensionless metric (e.g., condition number or minimum singular value) that can be difficult to interpret in terms of physical estimation accuracy.

- Linear error propagation via the Jacobian (new idea)
A good middle ground which propagates known input noise in the proximal wrench through the inverse Jacobian J-1 to estimate error bounds on force parameters F1s1θ1F2s2θ2. This method remains fast because it relies on only the forward model. Unlike SVD metrics, it produces parameter-wise error estimates, giving direct insight into the uncertainty of each force parameter F1s1θ1F2s2θ2 that we can directly minimize.

- Benefits of switching from SVD to linear error propagation

  - It produces results with physical units: I can report estimation accuracy in Newtons, meters, and radians rather than dimensionless singular values.

  - It solves the σ_min vs. condition number debate entirely: I am directly optimizing the thing both metrics were trying to approximate.

  - It gives me per-parameter diagnostics for free: I can see exactly how accurate each parameter [F1,s1,theta1,F2,s2,theta2] is.

Should we use condition number instead of "smallest singular value"?

- Using linear error propagation solves this issue as we no longer require either.

How do we pick noise for our proximal wrench?

- A simple approach is to use the ATI sensor resolution values from the spec sheet as the assumed measurement noise levels.

- For the purpose of design optimization, it doesn't matter what noise magnitudes I use, as long as the relative proportions between channels are roughly right.

- Even if real-world noise ends up being larger due to effects such as hysteresis or unmodeled disturbances, the geometry optimized using the spec-sheet resolutions will still be the best configuration for minimizing estimation error. The only difference is that the absolute error level in practice may be higher than predicted.

Things to think about:

Under what conditions would a robot result in only 5 measurements of wrench?

- The Cylindrical Design (Symmetric)

  - Planar Bending: When the steering and the applied force are in the same plane, torsion is mathematically zero.

  - Out-of-Plane Bending: When the steering and force are in different planes (e.g., steering up, force to the side), you get a small, torsional effect.

  - Problem: The torsional effect is very small and not very informative.

- For Optimized Design (Asymmetric / Elliptical)

  - Coupled Response: Whenever there is bending of any kind, the optimized geometry induces a large, amplified torsional effect. This is due to the elliptical segments.

  - Why this is better: By forcing the beam to twist as it bends, the strong torsion signal provides “richer” information about the forces applied.

Should tendon tensions be included in the Jacobian?

- Tendon tensions are actively held constant by the motors, so they do not change in response to external forces.

- A sensor reading that remains static in the presence of a disturbance provides zero information about that disturbance.

- Consequently, tendon tension serves as a configuration parameter (defining the backbone's stiffness) rather than a measurement variable (observing the applied external force).

- In contrast, motor encoder values must change to maintain this constant tension. Therefore, the change in cable length Δl provides information on the applied force.

- So ideally, Δl should be included in the Jacobian.

- But I don’t have a model that maps applied force to Δl or vice versa. So, this is a limitation of the method.

How many point forces can we actually estimate?

- Theoretical Limit (6 Measurements)

  - 6 F/T sensor inputs: [Tx, Ty, Tz, Fx, Fy, Fz]

  - Each contact needs 3 parameters (F, s, θ)

  - Can resolve: 2 point forces (2 × 3 = 6 unknowns)

- Expanded Limit (9 Measurements)

  - Add cable length changes: [Δl₁, Δl₂, Δl₃]

  - Creates 9 × 6 system

  - Can resolve: 3 point forces (3 × 3 = 9 unknowns)

- Practical Reality (Ill-Conditioning)

  - Estimating 3 forces is nearly impossible

  - Even tiny sensor noise creates massive errors so solution becomes unusable

References

[1] Efficient Force Estimation for Continuum Robot.pdf

[2] Estimating_Forces_Along_Continuum_Robots.pdf

[3] Profile and contact force estimation of cable-driven continuum robots in presence of obstacles