Experimental Procedure

- System Initialization: The ATI force/torque sensor, microcontroller, data logging software, and motor control script were powered on and initialized.

- Configuration: Loading cables were attached to the robot at arc lengths s1 and s2. The test rig gimbal arms were pre-positioned so that the loading cables create approximate angles θ1 and θ2. Loading cables were attached to the robot at arc lengths s1 and s1.

θ

y-axis

s

- Calibration: With all cables slack to prevent residual loading, the ATI sensor was tared to zero.

- Actuation: Target tendon tensions τ1, τ2, τ3 and external loads  F1 and  F2 were entered in the motor control script, triggering the winches to tension the cables automatically.

- Geometric Alignment: Upon reaching steady-state equilibrium, the gimbal arms were manually adjusted to correct deformation-induced shifts. This ensured the loading cables remained perpendicular to the backbone at their points of contact and aligned with target angles θ1 and θ2 in the loaded state.

θ

y-axis

90°

- Re-tensioning: To mitigate hysteresis induced by these manual adjustments, the motors were relaxed, and the calibration and actuation steps were repeated.

- Data Acquisition: Once the robot reached steady-state equilibrium and cable alignment was visually confirmed, the proximal wrench was recorded.

- Iteration: This procedure was repeated for all test configurations.